Name                 : Vizanth V

Reg No              :192512351

Course name     : Advanced Wireless Communincation

Course code      : ECA5610

MATLAB Simulation of Cellular Coverage, Cell Splitting and Handoff in a Multi-Cell Network

Aim:

# To model a simplified hexagonal cellular network and study how cell radius affects coverage, estimated user capacity and handoff as a mobile crosses a cell boundary.

# Objectives:

1.  Model adjacent hexagonal cell coverage and identify cell centers and boundaries.

2.  Estimate the number of cells required to cover a fixed service area for different cell radii.

3.  Compare estimated users served under an explicit per-cell capacity assumption.

4.  Simulate a mobile moving across a cell boundary and identify the serving-cell transitionSOFTWARE USED:

  - MATLAB

# Program :

# clear; clc; close all;

# % Baseline assumptions

# serviceAreaKm2 = 5;

# radiiKm = [1.0 0.5];

# usersPerCell = 30;

# hexArea = @(R) (3*sqrt(3)/2) .* R.^2;

# cellAreaKm2 = hexArea(radiiKm);

# cellCount = ceil(serviceAreaKm2 ./ cellAreaKm2);

# estimatedUsers = cellCount .* usersPerCell;

# capacityTable = table(radiiKm(:), cellAreaKm2(:), cellCount(:), ...

# repmat(usersPerCell,numel(radiiKm),1), estimatedUsers(:), ...

# 'VariableNames', {'Radius_km','CellArea_km2','EstimatedCells', ...

# 'UsersPerCell','IllustrativeUsers'});

# disp(capacityTable);

# % Plot a 7-cell cluster: central cell plus six neighbors

# R = radiiKm(1);

# centers = [0 0; 1.5*R sqrt(3)*R/2; 1.5*R -sqrt(3)*R/2; ...

# 0 sqrt(3)*R; 0 -sqrt(3)*R; -1.5*R sqrt(3)*R/2; ...

# -1.5*R -sqrt(3)*R/2];

# theta = (0:6)*pi/3; % flat-top hexagon vertices

# figure; hold on; axis equal; grid on;

# for k = 1:size(centers,1)

# plot(centers(k,1)+R*cos(theta), centers(k,2)+R*sin(theta), 'k-');

# text(centers(k,1),centers(k,2),sprintf('Cell %d',k), ...

# 'HorizontalAlignment','center');

# end

# xlabel('x (km)'); ylabel('y (km)');

# title('Idealized Seven-Cell Hexagonal Cluster');

# hold off;

# % Mobile moves across the boundary between centers at x=0 and x=1.5R

# mobileX = linspace(-0.2*R,1.7*R,300);

# mobileY = zeros(size(mobileX));

# d1 = hypot(mobileX, mobileY);

# d2 = hypot(mobileX-1.5*R, mobileY);

# servingCell = ones(size(mobileX));

# servingCell(d2 < d1) = 2;

# handoffIndex = find(diff(servingCell) ~= 0,1,'first') + 1;

# figure; hold on; grid on;

# plot(mobileX,d1,'LineWidth',1.5);

# plot(mobileX,d2,'LineWidth',1.5);

# if ~isempty(handoffIndex)

# xline(mobileX(handoffIndex),'--r','Handoff decision');

# end

# xlabel('Mobile x-position (km)'); ylabel('Distance to cell center (km)');

# legend('Cell 1','Cell 2','Location','best');

# title('Nearest-Center Handoff Approximation');

# hold off;

# figure; stairs(mobileX,servingCell,'LineWidth',1.5); grid on;

# xlabel('Mobile x-position (km)'); ylabel('Serving-cell ID');

# title('Serving Cell Along the Mobile Path');

# if ~isempty(handoffIndex)

# fprintf('Geometric handoff at x = %.3f km\n',mobileX(handoffIndex));

# end

# Output:

+